1.1.

Identification

SECTION 1: Identification

| Product form | : | Substance |  |  |
| --- | --- | --- | --- | --- |
| Substance name | : | MACHINE POWDER FOR PARTS WASHING |  |  |
| Product code | : | LC22965 |  |  |

1.2.	Recommended use and restrictions on use

| Use of the substance/mixture | : Industrial use : : Not for food, car use |
| --- | --- |
| Recommended use |  |
| Restrictions on use |  |

1.3.	Supplier

BÖRFH KİMYA SAN VE TİC A.Ş

Hicriye Mahallesi Yukarı Sk No:36/A SAKARYA - TÜRKİYE

T + 90 532 053 20 27

info@borfh.com - www.borfh.com

1.4.	Emergency telephone number

| Country | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Turkey | Ulusal Zehir Merkezi (UZEM) Refik Saydam Hıfzıssıhha Merkezi Başkanlığı | Cemal Gürsel Cd. No: 18 Sıhhiye Çankaya 06590 Ankara | 114 | Information is provided to public and medical personnel on poisoning incidents via 114. |

2.1.

Classification of the substance or mixture

SECTION 2: Hazard(s) identification

GHS US classification

| Skin corrosion/irritation Category 2 | H315 | Causes skin irritation |
| --- | --- | --- |
| Serious eye damage/eye irritation Category 2A | H319 | Causes serious eye irritation |

Full text of H statements : see section 16

## 2.2.	GHS Label elements, including precautionary statements GHS-US labelling

| Hazard pictograms (GHS US) | : |  |
| --- | --- | --- |
| Signal word (GHS US) | : | Warning |
| Hazard statements (GHS US) | : | H315 - Causes skin irritation H319 - Causes serious eye irritation |
| Precautionary statements (GHS US) | : | P264 - Wash exposed skin thoroughly after handling. P280 - Wear eye protection, protective gloves. P302+P352 - IF ON SKIN: Wash with plenty of soap and water. P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P332+P313 - If skin irritation occurs: Get medical advice/attention. P337+P313 - If eye irritation persists: Get medical advice/attention. P362 - Take off contaminated clothing and wash before reuse. |

2.3.	Other hazards which do not result in classification

| Other hazards not contributing to the classification | : | None. |
| --- | --- | --- |

2.4.	Unknown acute toxicity (GHS US)

3.1.

Substances

SECTION 3: Composition/information on ingredients

Not applicable

Substance type	:  Mono-constituent

| Name | Product identifier | % | GHS-US classification |
| --- | --- | --- | --- |
| Sodium Carbonate, Anhydrous (Main constituent) | (CAS-No.) 497-19-8 | =>20 | Skin Irrit. 2, H315 Eye Irrit. 2A, H319 |

3.2.	Mixtures

Full text of hazard classes and H-statements : see section 16

Not applicable

4.1.

Description of first aid measures

SECTION 4: First-aid measures

| First-aid measures general | : | Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible). |
| --- | --- | --- |
| First-aid measures after inhalation | : | Allow affected person to breathe fresh air. Allow the victim to rest. |
| First-aid measures after skin contact | : | Wash with plenty of soap and water. Wash contaminated clothing before reuse. If skin irritation occurs: Get medical advice/attention. Get medical advice/attention. |
| First-aid measures after eye contact | : | Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persists. |
| First-aid measures after ingestion | : | Rinse mouth. Do NOT induce vomiting. Obtain emergency medical attention. |

4.2.	Most important symptoms and effects (acute and delayed)

| Potential Adverse human health effects and symptoms | : | Based on available data, the classification criteria are not met. |
| --- | --- | --- |
| Symptoms/effects after inhalation | : | Coughing. |
| Symptoms/effects after skin contact | : | Causes skin irritation. |
| Symptoms/effects after eye contact | : | Causes serious eye irritation. |
| Symptoms/effects after ingestion | : | Vomiting. Nausea. |

4.3.	Immediate medical attention and special treatment, if necessary

None.

5.1.

Suitable (and unsuitable) extinguishing media

SECTION 5: Fire-fighting measures

| Suitable extinguishing media | : | Foam. Dry powder. Carbon dioxide. Water spray. Sand. |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use a heavy water stream. |

| 5.2. Specific hazards arisi | ng from the chem | ical |
| --- | --- | --- |
| Fire hazard | : | Non combustible. |
| Explosion hazard | : | Not applicable. |
| Reactivity in case of fire | : | Reacts with (some) acids. |

5.3.	Special protective equipment and precautions for fire-fighters

| Firefighting instructions | : | Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire-fighting water from entering environment. |
| --- | --- | --- |
| Protection during firefighting | : | Do not enter fire area without proper protective equipment, including respiratory protection. |

6.1.

Personal precautions, protective equipment and emergency procedures

SECTION 6: Accidental release measures

| General measures | : | None. |
| --- | --- | --- |

    - For non-emergency personnel

| Emergency procedures | : | Evacuate unnecessary personnel. |
| --- | --- | --- |

    - For emergency responders

| Protective equipment | : | Equip cleanup crew with proper protection. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate area. |

6.2.	Environmental precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

6.3.	Methods and material for containment and cleaning up

| Methods for cleaning up | : | On land, sweep or shovel into suitable containers. Minimize generation of dust. Store away from other materials |
| --- | --- | --- |

6.4.	Reference to other sections

See Heading 8. Exposure controls and personal protection.

7.1.

Precautions for safe handling

SECTION 7: Handling and storage

| Precautions for safe handling | : | Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapor. |
| --- | --- | --- |
| Hygiene measures | : | Wash exposed skin thoroughly after handling. |

7.2.	Conditions for safe storage, including any incompatibilities

| Storage conditions | : | Keep only in the original container in a cool, well ventilated place away from : incompatible materials. Keep container closed when not in use. |
| --- | --- | --- |
| Incompatible products | : | Strong bases. Strong acids. |
| Incompatible materials | : | Sources of ignition. Direct sunlight. |

| SECTION 8: Exposure controls/personal protection |  |  |  |
| --- | --- | --- | --- |
| 8.1. Control parameters |  |  |  |
| Machine Powder for Part Washing |  |  |  |
| No additional information available |  |  |  |

8.2.	Appropriate engineering controls

| Appropriate engineering controls | : | Emergency eye wash fountains should be available in the immediate vicinity of any potential exposure. |
| --- | --- | --- |

8.3.	Individual protection measures/Personal protective equipment

Personal protective equipment:. Gloves. Safety glasses..

Personal protective equipment:. Gloves. Safety glasses.. Hand protection:. Wear protective gloves.. Eye protection:. Chemical goggles or safety glasses. Skin and body protection:. Wear suitable protective clothing. Respiratory protection:. Respiratory protection not required in normal conditions. Dust production: dust mask with filter type P3. Personal protective equipment symbol(s):. Other information:. Do not eat, drink or smoke during use..

9.1.

Information on basic physical and chemical properties

SECTION 9: Physical and chemical properties

| Appearance | : | White powder or lumps. |
| --- | --- | --- |
| Color | : | white |
| Odor | : | odorless |
| Odor threshold | : | No data available |
| pH | : | 14 |
| Melting point | : | No data available |
| Freezing point | : | No data available |
| Boiling point | : | 1600 °C |
| Flash point | : | No data available |
| Relative evaporation rate (butyl acetate=1) | : | No data available |
| Flammability (solid, gas) | : | Non flammable. |
| Vapor pressure | : | No data available |
| Relative vapor density at 20 °C | : | No data available |
| Relative density | : | No data available |
| Specific gravity / density | : | 2.53 g/cm³ |
| Molecular mass | : | 105.99 g/mol |
| Solubility | : | No data available |
| Log Pow | : | No data available |
| Auto-ignition temperature | : | No data available |
| Decomposition temperature | : | No data available |
| Viscosity, kinematic | : | No data available |
| Viscosity, dynamic | : | No data available |
| Explosion limits | : | No data available |
| Explosive properties | : | Not applicable. |
| Oxidizing properties | : | None. |

9.2.	Other information

| Other properties | : | No additional information available |
| --- | --- | --- |

10.1.	Reactivity

SECTION 10: Stability and reactivity

Reacts with (some) acids.

## 10.2.        Chemical stability

Not established.

## 10.3.        Possibility of hazardous reactions

Not established.

## 10.4.        Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

## 10.5.        Incompatible materials

Strong acids. Strong bases.

## 10.6.        Hazardous decomposition products

Fume. Carbon monoxide. Carbon dioxide.

11.1.	Information on toxicological effects

SECTION 11: Toxicological information

Acute toxicity (oral)	:  Not classified

Acute toxicity (dermal)	:  Not classified

Acute toxicity (inhalation)	:  Not classified

| Machine Powder for Part Washing |  |
| --- | --- |
| LD50 oral rat | 4090 mg/kg |
| ATE US (oral) | 4090 mg/kg body weight |

Skin corrosion/irritation	Skin corrosion/irritation                                             :  Causes skin irritation. pH: 11.6

Serious eye damage/irritation	Serious eye damage/irritation                                   :  Causes serious eye irritation. pH: 11.6

Respiratory or skin sensitization	:  Not classified

Germ cell mutagenicity	:  Not classified

Carcinogenicity	:  Not classified

Reproductive toxicity	:  Not classified

STOT-single exposure	:  Not classified

STOT-repeated exposure	:  Not classified

Aspiration hazard	:  Not classified

Viscosity, kinematic	:   No data available

Likely routes of exposure	:  Inhalation. Skin and eye contact.

Potential Adverse human health effects and symptoms

:  Based on available data, the classification criteria are not met.

Symptoms/effects after inhalation	:  Coughing. Symptoms/effects after skin contact	:  Causes skin irritation.

Symptoms/effects after eye contact	:  Causes serious eye irritation. Symptoms/effects after ingestion	:  Vomiting. Nausea.

Chronic symptoms	:  Not available.

12.1.	Toxicity

SECTION 12: Ecological information

| Machine Powder for Part Washing |  |
| --- | --- |
| LC50 fish 1 | 300 mg/l |
| EC50 Daphnia 1 | 265 mg/l |
| LC50 fish 2 | 740 mg/l |

## 12.2.	Persistence and degradability

| Dıesel DPF Cleaner Powder |  |
| --- | --- |
| Persistence and degradability | Not established. |

12.3.	Bioaccumulative potential

| Machine Powder for Part Washing |  |
| --- | --- |
| Bioaccumulative potential | Not established. |

## 12.4.	Mobility in soil

| Dıes Machine Powder for Part Washing |  |
| --- | --- |
| Ecology - soil | No (test)data on mobility of the substance available. |

12.5.	Other adverse effects

| Other information | : | Avoid release to the environment. |
| --- | --- | --- |

13.1.	Disposal methods

SECTION 13: Disposal considerations

| Waste disposal recommendations | : | Dispose in a safe manner in accordance with local/national regulations. |
| --- | --- | --- |
| Ecology - waste materials | : | Avoid release to the environment. |

# SECTION 14: Transport information

## Department of Transportation (DOT)

| In accordance with DOT Not regulated |  |
| --- | --- |
| SECTION 15: Regulatory information |  |  |  |
| 15.1. US Federal regulations |  |  |  |
| Machine Powder for Part Washing |  |  |  |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |  |
| SARA Section 311/312 Hazard Classes |  | Health hazard - Skin corrosion or Irritation Health hazard - Serious eye damage or eye irritation |  |

All components of this product are listed, or excluded from listing, on the United States Environmental Protection Agency Toxic Substances Control Act (TSCA) inventory

## 15.2. International regulations	s

No additional information available

National regulations

No additional information available

## 15.3. US State regulations

California Proposition 65 - This product does not contain any substances known to the state of California to cause cancer, developmental and/or reproductive harm

# SECTION 16: Other information

Revision date; :; 11/09/2021. Other information; :; None.. Full text of H-phrases: see section 16:.

| H315 | Causes skin irritation |  |  |
| --- | --- | --- | --- |
| H319 | Causes serious eye irritation |  |  |
| NFPA health hazard | : | 1 - Materials that, under emergency conditions, can cause significant irritation. |  |
| NFPA fire hazard | : | 0 - Materials that will not burn under typical fire conditions, including intrinsically noncombustible materials such as concrete, stone, and sand. |  |
| NFPA reactivity | : | 0 - Material that in themselves are normally stable, even under fire conditions. |  |
| Hazard Rating |  |  |  |
| Health | : | 1 Slight Hazard - Irritation or minor reversible injury possible |  |
| Flammability | : | 0 Minimal Hazard - Materials that will not burn |  |
| Physical | : | 0 Minimal Hazard - Materials that are normally stable, even under fire conditions, and will NOT react with water, polymerize, decompose, condense, or self-react. Non-Explosives. |  |
| Personal protection | : | B - Safety glasses, Gloves |  |

SDS US Börfh Kimya Inc

Information in this SDS is from available published sources and is believed to be accurate. No warranty, express or implied, is made and Börfh Kimya Inc assumes no liability resulting from the use of this SDS. The user must determine suitability of this information for his application.