1.1.

Product identifier

SECTION 1: Identification of the substance/mixture and of the company/undertaking

Product form	:  Mixture

Product name	:  Brake Fluid DOT 3

1.2.	Relevant identified uses of the substance or mixture and uses advised against

Use of the substance/mixture	:  Brake fluids. For specific application advice see appropriate Technical Data Sheet or consult our company    representative.

1.3.	Details of the supplier of the safety data sheet

BÖRFH KİMYA SAN VE TİC A.Ş

Hicriye Mahallesi Yukarı Sk No:36/A                           SAKARYA - TÜRKİYE

T + 90 532 053 20 27

info@borfh.com - www.borfh.com

1.4.	Emergency telephone number

| Country | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Turkey | Ulusal Zehir Merkezi (UZEM) Refik Saydam Hıfzıssıhha Merkezi Başkanlığı | Cemal Gürsel Cd. No: 18 Sıhhiye Çankaya 06590 Ankara | 114 | Information is provided to public and medical personnel on poisoning incidents via 114. |

2.1.

Classification of the substance or mixture

SECTION 2: Hazards identification

## Classification according to Classification, Labelling and Packaging of Substances and Mixtures (SEA) Regulation published in the Official Journal numbered 28848 on December 11, 2013.

GHS Classification REPRODUCTIVE TOXICITY (Unborn child) - Category 2

2.2.	Label elements

Classification according to Classification, Labelling and Packaging of Substances and Mixtures (SEA) Regulation published in the Official Journal numbered 28848 on December 11, 2013.

Symbol

Signal word			 Warning

Hazard statements                               H315 Skin corrosion/irritation, Category 2

Precautionary statements

General 			Read label before use. Keep out of reach of children. If medical advice is needed, have product container or label at hand.

Prevention			P201 - Obtain special instructions before use.

P202 - Do not handle until all safety precautions have been read and understood.

P280 - Wear protective gloves. Wear eye or face protection. Wear protective clothing. Response

P308 + P313 - IF exposed or concerned: Get medical attention. Storage

P405 - Store locked up. Disposal

P501 - Dispose of contents and container in accordance with all local, regional, national and international regulations.

2.3.	Other hazards

Other hazards not contributing to the classification

No additional information available

3.1.

Substances

SECTION 3: Composition/information on ingredients

Not applicable

3.2.	Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| 2-(2-(2-butoxyethoxy)ethoxy)ethanol 2,2' -oxybisethanol 2-(2-butoxyethoxy)ethanol 2-(2-methoxyethoxy)ethanol | (CAS-No.) 143-22-6 (CAS-No.) 111-46-6 (CAS-No.) 112-34-5 (CAS-No.) 111-77-3 | ≥10 - ≤25 <10 ≤3 <3 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 STOT SE 3, H335 |

This mixture does not contain any substances to be mentioned according to the Regulation on Safety Data Sheets Regarding Hazardous Substances and Mixtures published in the Official Journal numbered 29204 on December 13, 2014

4.1.

Description of first aid measures

SECTION 4: First aid measures

First-aid measures after inhalation	:  No specific measures are necessary. First-aid measures after skin contact	:  Wash skin with plenty of water.

4.2.	Most important symptoms and effects, both acute and delayed

First-aid measures after eye contact	:  If eye irritation persists: Get medical advice/attention. First-aid measures after ingestion	:  Call a poison center or a doctor if you feel unwell.

No additional information available

4.3.	Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

5.1.

Extinguishing media

SECTION 5: Firefighting measures

Suitable extinguishing media	:  In case of fire, use foam, dry chemical or carbon dioxide extinguisher or spray.

5.2.	Special hazards arising from the substance or mixture

Hazardous decomposition products in case of fire

:  Combustion products may include the following: carbon oxides (CO, CO2) (carbon monoxide, carbon dioxide)

5.3.	Advice for firefighters

Protection during firefighting	:  Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing.

6.1.

Personal precautions, protective equipment and emergency procedures

SECTION 6: Accidental release measures

## For non-emergency personnel

Emergency procedures	:  No action shall be taken involving any personal risk or without suitable training. Evacuate surrounding areas. Keep unnecessary and unprotected personnel from entering. Do not touch or walk through spilt material. Put on appropriate personal protective equipment. Floors may be slippery; use care to avoid falling

## For emergency responders

Protective equipment	:  If specialised clothing is required to deal with the spillage, take note of any information in Section 8 on suitable and unsuitable materials. See also the information in "For non-emergency personnel".

6.2.	Environmental precautions

Avoid dispersal of spilt material and runoff and contact with soil, waterways, drains and sewers. Inform the relevant authorities if the product has caused environmental pollution (sewers, waterways, soil or air).

6.3.	Methods and material for containment and cleaning up

Small spill :  Stop leak if without risk. Move containers from spill area. Absorb with an inert material and place in an appropriate waste disposal container. Dispose of via a licensed waste disposal contractor.

Large spill :  Stop leak if without risk. Move containers from spill area. Prevent entry into sewers, water courses, basements or confined areas.

Contain and collect spillage with noncombustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container

for disposal according to local regulations. Dispose of via a licensed waste disposal contractor.

6.4.	Reference to other sections

For further information refer to section 13.

7.1.

Precautions for safe handling

SECTION 7: Handling and storage

Precautions for safe handling	:  Ensure good ventilation of the work station. Wear personal protective equipment.

Hygiene measures	:  Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

7.2.	Conditions for safe storage, including any incompatibilities

Storage conditions	:  Store in a well-ventilated place. Keep cool.

7.3.	Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1.

Control parameters

No additional information available

8.2.	Exposure controls

Appropriate engineering controls	:  Ensure good ventilation of the work station. Hand protection	:  Protective gloves

Eye protection	:  Safety glasses

Respiratory protection	:  In case of insufficient ventilation, wear suitable respiratory equipment

Environmental exposure controls	:  Avoid release to the environment.

9.1.

Information on basic physical and chemical properties

SECTION 9: Physical and chemical properties

Physical state	:  Liquid

Colour	:  Petrol Green

Odour	:  Characteristic

Odour threshold	:   No data available

pH	:   7.5 to 9

Relative evaporation rate (butylacetate=1)	:   No data available Melting point	:  Not applicable

Freezing point	:   No data available

Boiling point	:   >=210°C

Flash point	:   Open cup: 144°C (291.2°F) [Cleveland.]

Auto-ignition temperature	:   No data available

Decomposition temperature	:   No data available

Flammability (solid, gas)	:  Not applicable

Vapour pressure	:   No data available

Relative vapour density at 20 °C	:   No data available

Relative density	:   No data available

Density	:   1 >1000 kg/m³ (>1 g/cm³) at 30°C

Solubility	:   Soluble in water.

Log Pow	:   No data available

Viscosity, kinematic	:   Kinematic: 7.143 mm2 /s (7.143 cSt) at 40°C Kinematic: 1.986 mm2 /s (1.986 cSt) at 100°C

Viscosity, dynamic	:   No data available

Explosive properties	:   No data available

Oxidising properties	:   No data available

Explosive limits	:   No data available

9.2.	Other information

No additional information available

10.1.	Reactivity

SECTION 10: Stability and reactivity

The product is non-reactive under normal conditions of use, storage and transport.

## 10.2.	Chemical stability

Stable under normal conditions.

## 10.3.	Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

## 10.4.	Conditions to avoid

None under recommended storage and handling conditions (see section 7).

## 10.5.	Incompatible materials

No additional information available

## 10.6.	Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced.

11.1.	Information on toxicological effects

SECTION 11: Toxicological information

| Name | Category | Route of exposure | Target organs |
| --- | --- | --- | --- |
| 2,2' -oxybisethanol | Category 2 | Oral | Not determined |

Information on likely routes of exposure :  Routes of entry anticipated: Dermal, Inhalation.

Potential acute health effects

Eye contact No known significant effects or critical hazards.

Inhalation Vapour inhalation under ambient conditions is not normally a problem due to low vapour pressure

Skin contact No known significant effects or critical hazards.

Diethylene glycol: Ingestion of diethylene glycol can cause metabolic acidosis, kidney damage, central nervous system depression, and convulsions. The estimated human lethal dose is approximately 100 ml (3.4 ounces for an adult).

12.1.	Toxicity

SECTION 12: Ecological information

Ecology - general	:  The product is not considered harmful to aquatic organisms nor to cause long-term adverse effects in the environment.

Acute aquatic toxicity	:  Not classified

Chronic aquatic toxicity	:  Not classified

## 12.2.	Persistence and degradability

No additional information available

## 12.3.	Bioaccumulative potential

| Bioaccumulative potential | No additional information available |
| --- | --- |

12.4.	Mobility in soil

| Mobility in soil | No additional information available |
| --- | --- |

12.5.	Results of PBT and vPvB assessment

No additional information available

## 12.6.	Other adverse effects

Ozone	:  Not classified

Other adverse effects	:  No additional information available

13.1.	Waste treatment methods

SECTION 13: Disposal considerations

Waste treatment methods	:  Dispose of contents/container in accordance with licensed collector’s sorting instructions.

# SECTION 14: Transport information

In accordance with ADR / RID / IMDG / IATA / ADN

| ADR | IMDG |  | IATA ADN RID |  |  |
| --- | --- | --- | --- | --- | --- |
| 14.1. | UN number |  |  |  |  |
| Not regulated for transport |  |  |  |  |  |
| 14.2. | UN proper shipping name |  |  |  |  |
| Not applicable |  | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.3. | Transport hazard class(es) |  |  |  |  |
| Not applicable |  | Not applicable | Not applicable | Not applicable | Not applicable |
| Not applicable |  | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. | Packing group |  |  |  |  |
| Not applicable |  | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. | Environmental hazards |  |  |  |  |
| Dangerous for the environment : No |  | Dangerous for the environment : No Marine pollutant : No | Dangerous for the environment : No | Dangerous for the environment : No | Dangerous for the environment : No |
| No supplementary information available |  |  |  |  |  |

## 14.6.	Special precautions for user

- Overland transport

No data available

## Transport by sea

No data available

## Air transport

No data available

## Inland waterway transport

No data available

## Rail transport

No data available

## 14.7.	Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

15.1.	Safety, health and environmental regulations/legislation specific for the substance or mixture

SECTION 15: Regulatory information

Not applicable

## 15.1.1.	National regulations

This product doesn’t contain any substances that is controlled or prohibited for use according to the Regulation for Reduction of Ozone Depleting Substances published in the Official Journal numbered 27052 on November 12, 2008.

Regulatory reference	:  Regulation on Test Methods that will be Applied to Determine the Physicochemical, Toxicological and Ecotoxicological Properties of Substances and Mixtures published in the Official Journal numbered 28848 on December 11, 2013

Regulation on Health and Safety Precautions When Working with Chemical Substances published in the Official Journal numbered 28733 on August 12, 2013

Regulation on Inventory and Control of Chemicals published in the Official Journal numbered 27092 on December 26, 2008

Regulation on Restriction and Prohibition of Hazardous Substances and Mixtures published in the Official Journal numbered 27092 on December 26, 2008

Communiqué on Detergents and Surfactants Used in Detergents published in the Official Journal numbered 27092 on December 26, 2008.

# SECTION 16: Other information

Safety Data Sheet author's

Name	:	İrem BEKTAŞ KART

Certificate number	:	GBF-A-0-2732

Contact information	:

SDS Turkey

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product