SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1.	Product identifier

Product form	: Mixture

Product name	: TIRE REPAIR KIT

Vaporizer	: Aerosol

1.2.	Relevant identified uses of the substance or mixture and uses advised against

Use of the substance/mixture	: Used for all personal auto, motorcycles, bicycles, light commercial vehicles or caravans.

1.3.	Details of the supplier of the safety data sheet

BÖRFH KİMYA SAN VE TİC A.Ş

Hicriye Mahallesi Yukarı Sk No:36/A                           SAKARYA - TÜRKİYE

T + 90 309 99 51 - 52

info@borfh.com - www.borfh.com

1.4.	Emergency telephone number

| Country | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Turkey | Ulusal Zehir Merkezi (UZEM) Refik Saydam Hıfzıssıhha Merkezi Başkanlığı | Cemal Gürsel Cd. No: 18 Sıhhiye Çankaya 06590 Ankara | 114 | Information is provided to public and medical personnel on poisoning incidents via 114. |

SECTION 2: Hazards identification

2.1.	Classification of the substance or mixture

## Classification according to Classification, Labelling and Packaging of Substances and Mixtures (SEA) Regulation published in the Official Journal numbered 28848 on December 11, 2013.

Aerosol, Category 1	H222;H229 Skin corrosion/irritation, Category 1B H314

Full text of H-statements: see section 16

Adverse physicochemical, human health and environmental effects

: Pressurised container: May burst if heated. Extremely flammable aerosol. Causes severe skin burns and eye damage.

2.2.	Label elements

## Classification according to Classification, Labelling and Packaging of Substances and Mixtures (SEA) Regulation published in the Official Journal numbered 28848 on December 11, 2013.

Hazard pictograms (SEA)	:

GHS02	GHS05

Signal word (SEA)	: Danger

Hazardous ingredients	: ammonia, anhydrous

Hazard statements (SEA)	: H222 - Extremely flammable aerosol.

H314 - Causes severe skin burns and eye damage.

Precautionary statements (SEA)	: P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No

smoking.

P251 - Do not pierce or burn, even after use.

P260 - Do not breathe dust/fume/gas/mist/vapours/spray.

P264 - Wash hands, forearms and face with soap and water thoroughly after handling P280 - Wear protective gloves/protective clothing/eye protection/face protection.

P301+P330+P331 - IF SWALLOWED: Rinse mouth. Do NOT induce vomiting

P304+P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing.

2.3.	Other hazards

## Other hazards not contributing to the classification

No additional information available

SECTION 3: Composition/information on ingredients

3.1.	Substances

Not applicable

3.2.	Mixtures

| Name | Product identifier | % | Classification according to Classification, Labelling and Packaging of Substances and Mixtures (SEA) Regulation published in the Official Journal numbered 28848 on December 11, 2013. |
| --- | --- | --- | --- |
| butane | (CAS-No.) 106-97-8 (EC-No.) 203-448-7 (EC Index-No.) 601-004-00-0 | 20 - 30 | Flam. Gas 1, H220 Press. Gas |
| propane | (CAS-No.) 74-98-6 (EC-No.) 200-827-9 (EC Index-No.) 601-003-00-5 | 10 - 20 | Flam. Gas 1, H220 Press. Gas |
| isobutane | (CAS-No.) 75-28-5 (EC-No.) 200-857-2 (EC Index-No.) 601-004-00-0 | 1 - 10 | Flam. Gas 1, H220 Press. Gas |
| ammonia, anhydrous | (CAS-No.) 7664-41-7 (EC-No.) 231-635-3 (EC Index-No.) 007-001-00-5 | 0.1 - 5 | Flam. Gas 2, H221 Press. Gas Acute Tox. 3 (Inhalation), H331 Skin Corr. 1B, H314 Aquatic Acute 1, H400 |

Full text of H-statements: see section 16

SECTION 4: First aid measures

4.1.	Description of first aid measures

First-aid measures general	: IF exposed or concerned: Get medical advice/attention.

First-aid measures after inhalation	: Remove person to fresh air and keep comfortable for breathing.

First-aid measures after skin contact	: Wash skin with plenty of water. Take off contaminated clothing. Call a physician immediately. If

skin irritation or rash occurs:

First-aid measures after eye contact	: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing. If eye irritation persists: Get medical advice/attention.

4.2.	Most important symptoms and effects, both acute and delayed

First-aid measures after ingestion	: Rinse mouth. Do not induce vomiting. Call a physician immediately.

Symptoms/effects after skin contact	:  Burns. Symptoms/effects after eye contact	: Serious damage to eyes. Symptoms/effects after ingestion	: Burns.

4.3.	Indication of any immediate medical attention and special treatment needed

SECTION 5: Firefighting measures

5.1.	Extinguishing media

Treat symptomatically.

Suitable extinguishing media	: Water spray. Dry powder. Foam. Carbon dioxide.

5.2.	Special hazards arising from the substance or mixture

Fire hazard	: Extremely flammable aerosol.

Explosion hazard	: Pressurised container: May burst if heated.

Hazardous decomposition products in case of fire

: Toxic fumes may be released.

5.3.	Advice for firefighters

Protection during firefighting	: Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1.	Personal precautions, protective equipment and emergency procedures

## For non-emergency personnel

Emergency procedures	: Ventilate spillage area. No open flames, no sparks, and no smoking. Avoid contact with skin

and eyes. Do not breathe dust/fume/gas/mist/vapours/spray.

## For emergency responders

6.2.	Environmental precautions

Protective equipment	: Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection".

Avoid release to the environment.

6.3.	Methods and material for containment and cleaning up

Methods for cleaning up	: Mechanically recover the product.

6.4.	Reference to other sections

Other information	: Dispose of materials or solid residues at an authorized site.

For further information refer to section 13.

SECTION 7: Handling and storage

7.1.	Precautions for safe handling

Precautions for safe handling	: Ensure good ventilation of the work station. Keep away from heat, hot surfaces, sparks, open

flames and other ignition sources. No smoking. Do not spray on an open flame or other ignition source. Do not pierce or burn, even after use. Avoid contact with skin and eyes. Do not breathe dust/fume/gas/mist/vapours/spray. Wear personal protective equipment.

Hygiene measures	: Wash contaminated clothing before reuse. Do not eat, drink or smoke when using this product.

7.2.	Conditions for safe storage, including any incompatibilities

Always wash hands after handling the product.

Storage conditions	: Protect from sunlight. Do not expose to temperatures exceeding 50 °C/ 122 °F. Store locked up. Store in a well-ventilated place. Keep cool.

7.3.	Specific end use(s)

SECTION 8: Exposure controls/personal protection

8.1.	Control parameters

No additional information available

No additional information available

8.2.	Exposure controls

Appropriate engineering controls	: Ensure good ventilation of the work station. Hand protection	: Protective gloves

Eye protection	: Safety glasses

Skin and body protection	: Wear suitable protective clothing

Respiratory protection	: In case of insufficient ventilation, wear suitable respiratory equipment

Environmental exposure controls	: Avoid release to the environment.

SECTION 9: Physical and chemical properties

9.1.	Information on basic physical and chemical properties

Physical state	: Liquid

Colour	: Colourless

Odour	: Characteristic

Odour threshold	: No data available

pH	: 9 - 11

Relative evaporation rate (butylacetate=1)	: No data available Melting point	: Not applicable

Freezing point	:   No data available

Boiling point	:   No data available

Flash point	:   No data available

Auto-ignition temperature	:   No data available

Decomposition temperature	:   No data available

Flammability (solid, gas)	: Extremely flammable aerosol.

Vapour pressure	:   No data available

Relative vapour density at 20 °C	:   No data available

Relative density	:   No data available

Density	: 0.9 - 1 g/cm³

Solubility	:   No data available

Log Pow	:   No data available

Viscosity, kinematic	:   No data available

Viscosity, dynamic	:   No data available

Explosive properties	: Pressurised container: May burst if heated.

Oxidising properties	:   No data available

Explosive limits	:   No data available

9.2.	Other information

No additional information available

SECTION 10: Stability and reactivity

10.1.	Reactivity

10.2.	Chemical stability

Extremely flammable aerosol. Pressurised container: May burst if heated.

Stable under normal conditions.

10.3.	Possibility of hazardous reactions

10.4.	Conditions to avoid

No dangerous reactions known under normal conditions of use.

Avoid contact with hot surfaces. Heat. No flames, no sparks. Eliminate all sources of ignition.

10.5.	Incompatible materials

10.6.	Hazardous decomposition products

No additional information available

Under normal conditions of storage and use, hazardous decomposition products should not be produced.

SECTION 11: Toxicological information

11.1.	Information on toxicological effects

Acute toxicity	: Not classified

Skin corrosion/irritation	: Causes severe skin burns and eye damage. pH: 9 - 11

Serious eye damage/irritation	: Serious eye damage, category 1, implicit pH: 9 - 11

Respiratory or skin sensitisation	:   Not classified

Germ cell mutagenicity	:   Not classified

Carcinogenicity	:   Not classified

Reproductive toxicity	:   Not classified

STOT-single exposure	:   Not classified

STOT-repeated exposure	:   Not classified

Aspiration hazard	:   Not classified

| TIRE REPAIR KIT |  |
| --- | --- |
| Vaporizer | Aerosol |

SECTION 12: Ecological information

12.1.	Toxicity

Ecology - general	: Before neutralisation, the product may represent a danger to aquatic organisms.

Acute aquatic toxicity	:   Not classified

Chronic aquatic toxicity	:   Not classified

## Persistence and degradability

No additional information available

## Bioaccumulative potential

| TIRE REPAIR KIT |  |
| --- | --- |
| Bioaccumulative potential | No additional information available |

  - Mobility in soil

| TIRE REPAIR KIT |  |
| --- | --- |
| Mobility in soil | No additional information available |

  - Results of PBT and vPvB assessment

No additional information available

## Other adverse effects

Ozone	: Not classified

Other adverse effects	: No additional information available

# SECTION 13: Disposal considerations

## 13.1.	Waste treatment methods

Waste treatment methods	: Dispose of contents/container in accordance with licensed collector’s sorting instructions.

# SECTION 14: Transport information

In accordance with ADR / RID / IMDG / IATA / ADN

| ADR |  | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- | --- |
| 14.1. | UN number |  |  |  |  |
| 1950 |  | 1950 | 1950 | 1950 | 1950 |
| 14.2. | UN proper shipping name |  |  |  |  |
| AEROSOLS |  | AEROSOLS | Aerosols, flammable, containing substances in class 8, packing group iii | AEROSOLS | AEROSOLS |
| Transport document description |  |  |  |  |  |
| UN 1950 AEROSOLS, 2.1 (8), (D) |  | UN 1950 AEROSOLS, 2.1 (8) | UN 1950 Aerosols, flammable, containing substances in class 8, packing group iii, 2.1 | UN 1950 AEROSOLS, 2.1 (8) | UN 1950 AEROSOLS, 2.1 (8) |
| 14.3. | Transport hazard class(es) |  |  |  |  |
| 2.1 (8) |  | 2.1 (8) | 2.1 (8) | 2.1 (8) | 2.1 (8) |
| 14.4. | Packing group |  |  |  |  |
| Not applicable |  | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. | Environmental hazards |  |  |  |  |
| Dangerous for the environment : No |  | Dangerous for the environment : No Marine pollutant : No | Dangerous for the environment : No | Dangerous for the environment : No | Dangerous for the environment : No |
| No supplementary information available |  |  |  |  |  |

## Special precautions for user

- Overland transport

Classification code (ADR)	: 5FC

Special provisions (ADR)	: 190, 327, 344, 625

Limited quantities (ADR)	: 1l

Excepted quantities (ADR)	: E0

Packing instructions (ADR)	: P207, LP200

Special packing provisions (ADR)	: PP87, RR6, L2

Mixed packing provisions (ADR)	: MP9

Transport category (ADR)	: 1

Special provisions for carriage - Packages (ADR)

Special provisions for carriage - Loading, unloading and handling (ADR)

Special provisions for carriage - Operation (ADR)

: V14

: CV9, CV12

: S2

Tunnel restriction code (ADR)	: D

## - Transport by sea

Special provisions (IMDG)	: 63, 190, 277, 327, 344, 381, 959

Packing instructions (IMDG)	: P207, LP200

Special packing provisions (IMDG)	: PP87, L2

EmS-No. (Fire)	:  F-D

EmS-No. (Spillage)	:  S-U

Stowage category (IMDG)	: None

Stowage and handling (IMDG)	: SW1, SW22

Segregation (IMDG)	: SG69

## - Air transport

PCA Excepted quantities (IATA)	: E0

PCA Limited quantities (IATA)	: Y203 PCA limited quantity max net quantity (IATA)	: 30kgG PCA packing instructions (IATA)	: 203

PCA max net quantity (IATA)	: 75kg

CAO packing instructions (IATA)	: 203

CAO max net quantity (IATA)	: 150kg

Special provisions (IATA)	: A145, A167, A802

ERG code (IATA)	: 10C

## - Inland waterway transport

Classification code (ADN)	: 5FC

Special provisions (ADN)	: 190, 327, 344, 625

Limited quantities (ADN)	:   1 L

Excepted quantities (ADN)	:  E0

Equipment required (ADN)	: PP, EP, EX, A

Ventilation (ADN)	: VE01, VE04

Number of blue cones/lights (ADN)	: 1

## - Rail transport

Classification code (RID)	: 5FC

Special provisions (RID)	: 190, 327, 344, 625

Limited quantities (RID)	:  1L

Excepted quantities (RID)	:  E0

Packing instructions (RID)	: P207, LP200

Special packing provisions (RID)	: PP87, RR6, L2

Mixed packing provisions (RID)	: MP9

Transport category (RID)	: 1

Special provisions for carriage – Packages (RID)

Special provisions for carriage - Loading, unloading and handling (RID)

: W14

: CW9, CW12

Colis express (express parcels) (RID)	: CE2

Hazard identification number (RID)	: 238

## Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1.	Safety, health and environmental regulations/legislation specific for the substance or mixture

## 15.1.1.	National regulations

This product doesn’t contain any substances that is controlled or prohibited for use according to the Regulation for Reduction of Ozone Depleting Substances published in the Official Journal numbered 27052 on November 12, 2008.

Regulatory reference	: Waste Management Regulation published in the Official Journal numbered 29314 on April 2, 2015

Regulation on Test Methods that will be Applied to Determine the Physicochemical, Toxicological and Ecotoxicological Properties of Substances and Mixtures published in the Official Journal numbered 28848 on December 11, 2013

Regulation on Health and Safety Precautions When Working with Chemical Substances published in the Official Journal numbered 28733 on August 12, 2013

Regulation on Inventory and Control of Chemicals published in the Official Journal numbered 27092 on December 26, 2008

Regulation on Restriction and Prohibition of Hazardous Substances and Mixtures published in the Official Journal numbered 27092 on December 26, 2008

Communiqué on Detergents and Surfactants Used in Detergents published in the Official Journal numbered 27092 on December 26, 2008.

SECTION 16: Other information

Full text of H-statements

| Acute Tox. 3 (Inhalation) | Acute toxicity (inhal.), Category 3 |
| --- | --- |
| Aquatic Acute 1 | Hazardous to the aquatic environment — Acute Hazard, Category 1 |
| Flam. Gas 1 | Flammable gases, Category 1 |
| Flam. Gas 2 | Flammable gases, Category 2 |
| Press. Gas | Gases under pressure |
| Skin Corr. 1B | Skin corrosion/irritation, Category 1B |

Safety Data Sheet author's

Name	:	İrem BEKTAŞ KART

Certificate number	:	GBF-A-0-2732

SDS Turkey

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product